Autoclave 110 L

Model: SQL1010C

Brand: Yamato

Origin: Japan

Technical Specifications:

System: Automatic high-pressure steam sterilization

Operating temperature range: 45 ~135°C/ 45 ~80°C (pre-heating) / 45 ~ 60°C (heat retention) /65 ~100°C (melting) / 105 ~135°C (sterilization)

Max. working pressure: 0.255 MPa

Lid (cover mechanism): Manual upward opening with an interlock for safety

Heater: 2000W x 2 pcs.

Exhaust valve: One exhaust valve and one slow release valve.

Option port for sample sensor (1/4), recorder (1/4) and connection to pressure gauge Cooling fan Axial fan motor.

Temp. controller: PID control by microprocessor.

Timer / Timer resolution: 0 or 1 min. to 99 hrs 59 min. / 1 minute

Operation mode: Instrument sterilization course, liquid sterilization course, sterilization and keep warm course, melting and keep warm course, Other functions: customer-programmed course, Key locking, presetting, saving, preheating, forced cooling, sample temperature sensor (option), pattern locking, up to 20 error log saving, display of accumulated working time / present time, ON-OFF beeping setting.

Safety device: Sensor failure detection, SSR short-circuit, broken heater wire, prevention of idle heating (liquid expansion type), alarm against the absence of a drain bottle, failure in locking the lid, memory error detection, pressure relief valve (0.255 MPa)

Internal dimensions of chamber: I.D.450 x 692 mm

Internal capacity: 80L-110L

Weight: 170kg

Power source: AC220V 19A

Included accessories: 2 pcs. stainless steel mesh basket (Dia.424 x H200 mm) 3 pcs. stainless steel mesh basket (Dia.424 x H200 mm)

Terms & Condition:

Manufacturer Authorization must be submitted with the tender and strictly not accepted any other third-party authorization.

Warranty/Guarantee: 01 (One) years from the date of installation

Country of Origin: USA/Japan/UK or Equivalent.