Autoclave 32 liter

Brand: Yamato Scientific

Model: SN300

Country: Japan

Technical Specification

Should be Automatic high-pressure steam sterilization

Operating temperature range: 45～135℃

Temperature function:

45～80℃ (Preheating temperature)

45～60℃ (Heat-retention process)

65～100℃ (Solution process)

105～135℃ (Sterilization)

Working pressure: 0.255MPa

Should have Manual upward opening with an interlock for safety lid

Heater:  800W×2 units heater unit

Option:  Exhaust valve One exhaust valve and one slow release valve with Axial fan motor Cooling fan

Controller: PID control by microprocessor Temp. controller

Display: digital display for Temperature and time

Timer: 0 or 1 min. to 99 hrs 59 min. / 1 min. Timer / Timer resolution

Should have Instrument sterilization course, Liquid sterilization course, Sterilization and keep warm course, Melting and keep warm course, Customer-programmed course operation function

Should have Key locking, Presetting, Saving, Preheating, forced cooling, Sample temperature sensor (option), Pattern locking, Up to 20 error log saving, Display of accumulated working time / Present time, ON-OFF beeping setting function

Safety: Sensor failure detection, SSR short-circuit, Broken heater wire, Prevention of idle heating (Liquid expansion type), Alarm against the absence of a drain bottle, Failure in locking the lid, Memory error detection, Pressure relief valve (0.255 MPa).

Should have Internal dimensions φ300×445mm

Should have Internal capacity 32L

Should have Power Source AC110-240 V

Country of Origin: USA/Japan/Italy/Germany or Equivalent.

Manufacturer Authorization must be submitted with the tender and strictly not accepted any other third-party authorization.

Warranty/Guarantee: 01 (one) year from the date of installation.