Clean bench or Vertical Laminar flow

Model: ADE101

Brand: Yamato

Origin: Japan

Specifications:

Air flow direction: Vertical direction (down flow), Forced circulation exhaust type (weak negative pressure)

Cleanliness: Class 100 (JIS Class 5)

Blow-off air speed (m/sec): Approx. 0.35 at early stage, Approx. 0.17 final

Illuminance (lx): 1000 (in the center of working face)

Load bearing of working face: 50kg (static load)

Air volume Work space: Approx.13 m3/min

Air volume Exhaust: Approx.1.5 m3/min

Exterior material: Cold roll steel sheet (chemical-resistant coating)

Interior material	Stainless steel sheet: (SUS304)

Working face: stainless steel (SUS304), wire drawing processing

Front door frame: Toughened glass 5mm, open and close up and down, spare 5mm opening when fully closed

HEPA Filter (mm) Dust collection efficiency: 0.3μm DOP scan test 99.99 % or more

HEPA Filter (mm) Main: 915 x 610 x 65 / 1pc

HEPA Filter (mm) Exhaust: 305 x 200 x 50 / 1pc

Socket: AC220V / 2pcs

Fluorescent lamp: 32W x 2pcs.

Blower: 150W x 1

Working surface level adjustment: Use adjusting foot (-18~+40mm)

Display of pressure difference: Differential pressure gauge

Gas burner: 1 pc / fire ignition type (can be used only when operating the machine)

Overcurrent protector: Earth leakage breaker 1pc

Optional item: Wind velocity controller: displays wind velocity and with wind adjust device attachment

Internal dimensions (H mm): 720mm

External dimensions (W×D×H mm): 980×770×1820mm

Weight:190kg

Manufacturer Authorization must be submitted with the tender and strictly not accepted any other third-party authorization.

Warranty/Guarantee: 01 (One) years from the date of installation

Country of Origin: USA/Japan/UK or Equivalent.